IPV4 Static routing: Router 1

enable

configure terminal

hostname AyushaAdhikari

interface GigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 192.168.4.2

ip route 192.168.3.0 255.255.255.0 192.168.4.2

Router 2:

enable

configure terminal

interface GigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.4.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/2

ip address 192.168.5.1 255.255.255.0

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 192.168.4.1

ip route 192.168.3.0 255.255.255.0 192.168.5.2

Router 3

enable

configure terminal

interface GigabitEthernet 0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.5.2 255.255.255.0

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 192.168.5.1

ip route 192.168.2.0 255.255.255.0 192.168.5.1

ipv4 dynamic routing:

Router 0:

enable

configure terminal

hostname AyushaAdhikari

interface GigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

exit

router rip

network 192.168.1.0

network 192.168.4.0

Router 1:

enable

configure terminal

hostname AyushaAdhikari

interface GigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.4.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/2

ip address 192.168.5.1 255.255.255.0

no shutdown

exit

router rip

network 192.168.2.0

network 192.168.4.0

network 192.168.5.0

Router 2:

enable

configure terminal

hostname AyushaAdhikari

interface GigabitEthernet 0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet 0/1

ip address 192.168.5.2 255.255.255.0

no shutdown

exit

router rip

network 192.168.3.0

network 192.168.5.0

Lab to conduct IPv6 Auto Configuration

enable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2001::/64 eui-64

no shutdown

exit

Lab to conduct IPV6 Static Routing

Router0 (Left) — Clean Commandsenable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:1::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2001::19/64

no shutdown

exit

ipv6 route 2000:2::/64 2001::20

ipv6 route 2000:3::/64 2001::20

Router1 (Middle) — Clean Commandsenable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:2::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2001::10/64

no shutdown

exit

interface GigabitEthernet 0/2

ipv6 enable

ipv6 address 2002::10/64

no shutdown

exit

ipv6 route 2000:1::/64 2001::10

ipv6 route 2000:3::/64 2002::20

Router2 (Right) — Clean Commandsenable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:3::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2002::20/64

no shutdown

exit

ipv6 route 2000:1::/64 2002::10

ipv6 route 2000:2::/64 2002::10

Lab to conduct IPv6 Dynamic Routing

IPV6 Dynamic IP Configuration:

Router0 (Left) — Clean Commands

enable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:1::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2001::10/64

no shutdown

exit

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 rip net1 enable

exit

interface GigabitEthernet 0/1

ipv6 rip net1 enable

exit

Router1 (Middle) — Clean Commands

enable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:2::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2001::20/64

no shutdown

exit

interface GigabitEthernet 0/2

ipv6 enable

ipv6 address 2002::10/64

no shutdown

exit

interface GigabitEthernet 0/0

ipv6 rip net1 enable

exit

interface GigabitEthernet 0/1

ipv6 rip net1 enable

exit

interface GigabitEthernet 0/2

ipv6 rip net1 enable

exit

Router2 (Right) — Clean Commands

enable

configure terminal

hostname AyushaAdhikari

ipv6 unicast-routing

interface GigabitEthernet 0/0

ipv6 enable

ipv6 address 2000:3::1/64

no shutdown

exit

interface GigabitEthernet 0/1

ipv6 enable

ipv6 address 2002::20/64

no shutdown

exit

interface GigabitEthernet 0/0

ipv6 rip net1 enable

exit

interface GigabitEthernet 0/1

ipv6 rip net1 enable

exit